Int. No. 473

By Council Members Williams, Vacca, Levine, Espinal, Lancman, Vallone, Richards, Koo, Crowley, Chin, Dickens, Gibson and Miller

..Title

A Local Law to amend the New York city building code, in relation to strengthening scaffolding requirements.

..Body

Be it enacted by the Council as follows:

Section 1. Section 3314.3.2 of the New York city building code, as added by local law number 141 for the year 2013, is amended to read as follows:

3314.3.2 Suspended scaffolds. Suspended scaffolds shall be designed by a registered design professional.

Exceptions:

[1. Design is not required for a single tier non-adjustable suspended scaffold whose platform is 40 square feet (12 192mm) or less in size.]

[2.] 1. In lieu of a registered design professional, a two-point, single tier, suspended scaffold may be designed by a licensed rigger provided:

[2.1] 1.1 The scaffold or scaffold outrigger beam or suspension member support structure is not anchored to the building or structure, other than tiebacks; and

[2.2] 1.2 The scaffold will be loaded or designed to be loaded, in excess of 75 pounds per square foot (355.15 kg/ m2); and either

[2.2.1] 1.2.1 The scaffold utilizes c-hooks; or

[2.2.2] 1.2.2 The distance from floor or roof on which the support structure is located to the top of the outrigger beam or suspension member support structure is less than 15 feet.

[3.] 2. In lieu of a registered design professional or a licensed rigger, a two-point single tier, suspended scaffold meeting the requirements of Item [2] 1 of these exceptions that is used exclusively for sign hanging work may be designed by a licensed sign hanger.

§2. Exception 2 of section 3314.4.1.1 of the New York city building code, as added by local law number 141 for the year 2013, is amended to read as follows:

2. The installation and removal of a suspended scaffold may be supervised by a competent person designated by the scaffold permit holder, or where there is no scaffold permit holder, designated by the scaffold controlling entity, provided that such scaffold is installed and removed in conjunction with:

2.1. The construction of a new building;

2.2. The full demolition of an existing building; or

2.3. The vertical or horizontal enlargement of an existing building.[;]

[2.4. The alteration, maintenance, or repair of a façade of a major building where a site safety plan is required by Section 3310.3.]

§3. Exception 2 of section 3314.4.2.1 of the New York city building code, as added by local law number 141 for the year 2013, is amended to read as follows:

2. In lieu of direct and continuing supervision by a licensed rigger, the use of a suspended scaffold may be supervised by a competent person designated by the scaffold controlling entity, provided such scaffold is used in conjunction with:

2.1. The construction of a new building;

2.2. The full demolition of an existing building; or

2.3. The vertical or horizontal enlargement of an existing building.[;]

[2.4. The alteration, maintenance, or repair of a façade of a major building where a site safety plan is required by Section 3310.3.]

§4. The exception to section 3314.4.3.4 of the New York city building code, as added by local law number 141 for the year 2013, is REPEALED.

§5. Exception 4 of section 3314.8 of the New York city building code, as added by local law number 141 for the year 2013, is REPEALED.

§6.  This local law shall take effect on October 1, 2014, except that the commissioner of buildings shall take such measures as are necessary for its implementation, including the promulgation of rules, prior to such effective date.

LS 2071

JW

7/31/14