Staff: 	Baaba Halm

Assistant Counsel to the Committee

Benjamin J. Goodman

Policy Analyst to the Committee

Wayne Miller

Policy Analyst to the Task Force on the Improvement of the Operations of the Department of Buildings

THE COUNCIL

REPORT OF THE INFRASTRUCTURE DIVISION

ROBERT NEWMAN, LEGISLATIVE DIRECTOR

COMMITTEE ON HOUSING AND BUILDINGS

Hon. Erik Martin Dilan, Chair

July 15, 2008

PROPOSED INT. NO. 796-A:	By: Council Member Lappin (in conjunction with the Mayor), Comrie, Gentile, Jackson, James, Koppell, Monserrate, Nelson, Palma, Recchia Jr., Seabrook, Stewart, Weprin and Gerson.

TITLE:	A Local Law to amend the administrative code of the city of New York, in relation to tower and climber cranes.

ADMINISTRATIVE CODE:	Amends title 28 by amending sections 3302.1 and 3319.

BACKGROUND AND ANALYSIS:

On July 15, 2008, the Committee on Housing and Buildings, chaired by Council Member Erik Martin Dilan, will conduct a hearing on Proposed Int. No. 796-A, A Local Law to amend the Administrative Code of the City of New York, in relation to tower and climber cranes.  In light of recent crane accidents and tragedies, the Council is concerned about the use of such equipment and the communication and coordination between the many workers involved in tower and climber crane erection, jumping, climbing, and dismantling operations. The Committee expects to hear testimony from representatives of the Department of Buildings, real estate professionals, including contractors and other persons interested in this legislation.

Proposed Int. No. 796-A

Bill section one would amend section 3302.1 of Chapter 33 of Title 28 of the Administrative Code of the City of New York (“Ad. Code”) by adding definitions related to “climbing,” “dismantling” and “erection” as such processes relate to tower and climber cranes.  “Climbing” would be defined as the raising or lowering of a tower or climber crane to different floors or levels of a building or structure.  “Dismantling” would be defined as the process of taking apart, piece by piece, in a specific sequence, the components of a crane and the definition of dismantling would also include “climbing” and “jumping”. “Erection” would be defined as the assembly and placement of crane sections and components into place, including all operations related to such assembly and would also include “climbing” and “jumping”.  “Jumping” is already defined in the Building Code.

Bill section two would amend the Building Code of the City of New York by adding a new section 3319.8 to provide special provisions related to the use of tower and climber cranes.  Section 3319.8.1 would require a licensed engineer, in conjunction with a licensed rigger, to prepare a plan for the erection, jumping, climbing and dismantling of a tower or climber crane.  Such plan must be in compliance with the tower or climber crane manufacturer’s recommendation for such processes where such recommendations exist and must be filed with the Department of Buildings (DOB) when a certificate for an on-site inspection is sought.  Erecting, jumping, climbing or dismantling of a tower or climber crane without the prior issuance of a certificate of on-site inspection by the  DOB’s Cranes and Derricks Unit would be prohibited.

This provision also mandates that plans for the erection, jumping, climbing or dismantling of a crane include: identification of all equipment to be used in such procedures; identification of the assemblies and sub-assemblies needed for the erection and dismantling of the crane equipment; location of the equipment, sidewalk sheds, surrounding buildings, protection for their roofs and the pick-up points, loads, and radius of the swing for all loads.  The safe load from the approved load radius chart must also be submitted for lift radius.

Additionally, the plan must include: a weight list of all proposed assemblies and sub-assemblies, in addition to the marking and certification of such weights; identification of the center of gravity for all asymmetrical components; information related to the sequence of all erecting, jumping, climbing and dismantling operations along with rigging materials; certification of the calibration of installed safety devices; identification  of other cranes or derricks within the lot line or on the street which will be used in such operations; a time schedule for which such operations are proposed to take place, and the names and contact information for the licensed rigger, crane safety coordinator and the company performing the erection, dismantling, climbing or jumping operations.

Section 3319.8.2 would require the general contractor of the construction site to hold a safety coordination meeting prior to an initial jump or climb, as well as the dismantling or initial jump down of a climber or tower crane. No work related to the erection, climbing, jumping or dismantling of the tower or climber crane may be performed without the safety coordination meeting having taken place. Such meeting must include the participation of the general contractor or designee, professional engineer of record for the crane or designee, licensed tower or climber crane rigger, crane safety coordinator, site safety manager or coordinator as required for major buildings and any other party deemed appropriate by the department.

Section 3319.8.3 would require a pre-jump safety meeting no more than 24-hours prior to each instance of a tower or climber crane jump or climb.  No work related to the jumping or climbing of the tower or climber crane may be performed without the pre-jump safety meeting having taken place.  Such meeting must include the participation of the general contractor or designee; professional engineer of record for the crane or designee; licensed tower or climber crane rigger; crane safety coordinator; site safety manager or coordinator as required for major buildings; crane operator and oiler; jumping crew and back-up personnel; flagman; signalman and communications personnel and any other party deemed appropriate by the department.

Section 3319.8.4 would mandate that the general contractor notify DOB at least 48-hours before any safety coordination meetings or pre-jump safety meetings are held.  No work related to the jumping or climbing of the tower or climber crane may be performed without prior notice of such meetings being provided to the Department.

Section 3319.8.5 mandates that safety coordination and pre-jump safety meetings cover topics related to: the scope of work; roles and responsibilities; rigging to be used and the specific sequence of operations; inspection of all rigging equipment, materials, and tools prior to work; review of all equipment, including but not limited to collars, ties and bolts; permit validity; qualifications and training of personnel; relevant weather warnings, and compliance with the manufacturer’s manual and softening mechanisms, if using nylon slings.

Section 3319.8.6 requires the general contractor, his or designee and/or the company erecting, jumping, climbing or dismantling a tower or climber crane to keep and maintain on the construction site a meeting log that includes the dates and times of all safety coordination and pre-jump meetings held; the names, titles and affiliations of all participants of such meetings; a summary of what was discussed at such meetings and the name of the person to whom specific tasks were assigned; a list of any decisions made at such meetings and certification of worker training as required under section 3319.8.2.

Section 3319.8.7 mandates that prior to the jumping or climbing of a tower or climber crane, the engineer of record for the crane must provide to DOB a certified report stating that the crane installation was inspected prior to the pre-jump safety meeting, the date of such inspection must be provided; that the crane is installed according to the plans approved by DOB and in accordance with manufacturer’s specifications; that the appropriate technical testing records for the crane, including torque, plumb, and magnetic particle reports comply with safety requirements and the manufacturing specifications and that no hazardous conditions or any other condition that might adversely affect the safety of the proposed erection, dismantling, climbing or jumping operations were found during such crane inspection.

Section 3319.8.8 provides that all crane erection, jumping, climbing and dismantling operations: (1) must have a licensed tower or climber crane rigger, and the crane safety coordinator or designee present on the job site during such operations; (2) cranes and derricks located within the lot line or on the street and used to erect, jump, climb or dismantle tower or climber cranes will be subject to certificate of on-site inspection requirements; (3) a DOB-approved load radius chart must be posted in the cabin of the crane; (4) a DOB-approved erection, jumping, climbing or dismantling procedure and sequence, with weights of assemblies and sub-assemblies clearly marked, must be given to the crane or derrick operator and to the rigger prior to commencement of any work; (5) a permit from New York City Department of Transportation is required for any tower or climber crane operations on any roadway, sidewalk or street; (6) all accepted or approved installed safety devices on a crane must have been calibrated within the preceding three months and (7) all safety devices on a tower or climber crane must be inspected by the crane operator as part of his/her inspection procedure.

Bill section three provides that this local law shall take effect thirty days from its enactment.